EE-111 Linear Circuit Anaylsis

End Semester Project Report

Title:  Home Automation System

Type: Voice Controlled

Submitted to: Engr. Ali Khalid

Submitted By:

| Group Members | CMS ID |
| --- | --- |
| Laveeza Fatima | 515521 |
| Ramisa Irum | 504703 |
| Ayesha Mohsin | 517660 |
| Fatima Habib | 509029 |

Table of Contents

- Introduction

- Objectives

- System Overview

- Methodology

  - Hardware

  - Software

  - Steps

- Implementation

- Results

- Challenges

- Future Work

- Conclusion

- Team Contribution

- References

1. Introduction

The advent of smart technologies has revolutionized how we interact with everyday objects. This project aims to develop a Voice Controlled Home Automation System, providing an efficient and user-friendly solution for controlling home appliances using voice commands. The system is particularly beneficial for individuals with mobility challenges, enhancing comfort and accessibility.

2. Objectives

- To design a voice-activated system capable of controlling various home appliances.

- To ensure reliability, low power consumption, and seamless integration with existing household devices.

- To improve user convenience and promote sustainable energy usage by avoiding unnecessary appliance operation.

3. System Overview

The system consists of three primary components:

- Voice Recognition Module: This module converts spoken commands into digital signals.

- Control Unit: Processes the signals and triggers the appropriate appliance.

- Appliances: Standard electrical devices connected to the automation system.

The communication between the components is facilitated via Bluetooth.

4. Methodology

Hardware:

- Microcontroller: Arduino Mega.

- Relay modules for appliance switching.

- Bluetooth Module: HC-05.

- Power Supply: DC adapter.

- APP named Arduino Bluetooth Contoller

- Google Assistant for voice input

- LEDs (acting as bulbs), DC Motor (acting as fan), and Servo Motor for door automation.

Software:

- Voice recognition using a compatible smartphone app.

- Programming environment: Arduino IDE.

- Communication protocol: Serial communication via Bluetooth.

Steps:

- Program the microcontroller to interpret voice commands received through the Bluetooth module (HC-05).

- Interface the microcontroller with relay circuits connected to appliances.

- Use relay modules to connect two LEDs simulating light bulbs and a DC motor as a fan.

- Directly connect a servo motor to the Arduino Mega for door automation with 90-degree rotation.

- Implement a control protocol to manage communication between the hardware and the smartphone/voice assistant.

- Test and troubleshoot the system to ensure reliability.

5. Implementation

The project was implemented over four phases:

- Designing the Circuit: Developed schematics integrating the microcontroller, relays, and the power supply.

- Programming the Microcontroller: Wrote and uploaded code for command interpretation and device control.

- System Integration: Connected hardware components and established communication protocols.

- Testing and Optimization: Ensured accuracy in voice command recognition and appliance control.

6. Results

The final system successfully:

- Recognized predefined voice commands.

- Operated connected appliances efficiently without latency.

- Controlled LEDs (bulbs), a DC motor (fan), and a servo motor (door) seamlessly.

- Allowed for additional appliance integration without altering the base design.

- Worked seamlessly within the typical Bluetooth coverage area.

7. Challenges

- Ensuring accurate voice recognition in noisy environments.

- Balancing energy efficiency with the need for active listening by the system.

- Integration with a wide variety of appliance types and models.

8. Future Work

To enhance the system's capabilities, future improvements could include:

- Adding compatibility with more voice assistants and languages.

- Incorporating advanced AI for personalized automation and predictive analytics.

- Implementing robust security protocols to prevent unauthorized access.

9. Conclusion

This Voice Controlled Home Automation System represents a significant step toward smarter and more accessible living spaces. The project achieved its objectives by delivering a functional, reliable, and scalable solution. With further development, it holds immense potential for widespread adoption.

10. Team Contribution

- Hardware Design and Assembly: [Laveeza Fatima & Ramisa Irum]

- Programming and Software Integration: [Fatima Habib & Ayesha Mohsin]

11. Code:

#include <Servo.h>

Servo myservo;  // create servo object to control a servo

void setup() {

myservo.attach(9);

myservo.write(0);

Serial.begin(9600);  // Start the serial communication

pinMode(2, OUTPUT);  // Set pin 2 as an output

pinMode(3, OUTPUT);  // Set pin 3 as an output

pinMode(4, OUTPUT);  // Set pin 4 as an output

pinMode(5, OUTPUT);  // Set pin 5 as an output

digitalWrite(4, LOW); // Initialize LED off

digitalWrite(5, LOW); // Initialize LED off

digitalWrite(2, LOW); // Initialize LED off

digitalWrite(3, LOW); // Initialize LED off

}

String voice;

void loop() {

if (Serial.available() > 0) {

voice = Serial.readString();  // Read the received string

voice.trim();                 // Remove any leading/trailing whitespace, \n, \r

Serial.println("Command: " + voice); // Debugging: print the command

}

if (voice == "turn on LED") {

digitalWrite(4, HIGH); // Turn on the LED

digitalWrite(5, HIGH);

Serial.println("LED is ON");

} else if (voice == "turn off LED") {

digitalWrite(4, LOW);  // Turn off the LED

digitalWrite(5, LOW);

Serial.println("LED is OFF");

}

else if (voice=="turn on fan"){

digitalWrite(2, HIGH); // Turn on the LED

Serial.println("FAN is ON");

}

else if (voice=="turn off fan"){

digitalWrite(2, LOW); // Turn on the LED

Serial.println("FAN is OFF");

}

else if (voice=="open door"){

myservo.write(90);

Serial.println("DOOR is OPEN");

}

else if (voice=="close door"){

myservo.write(0);

Serial.println("DOOR is close");

}

}

12. Simulations:

13. Circuit Pictures:

14: References:

- https://youtu.be/HG-W3-bLvpo?si=QKaoeJ4j_i0cfDRa

- https://youtu.be/gpB4600keWA?si=KIIi0BQ_gINl56Dl